# Tubes (0.5 or 1.5 ml) – purifying DNA Templates  (04 June 2025)

See “Science Aid Center” on Core website for additional information as indicated below.

- DNA recovery: Should be >80-90%; for safety factor, assume 70%.  Estimate required downstream volumes based on brightness of raw PCR products in a gel.  For example:

- Assume you ran 5 ul of a 25-ul PCR in an agarose gel, and estimated by the band brightness (see images at “How much DNA to use in a sequencing reaction?”) that you need 3-ul of raw product for a BigDye DNA Sequencing reaction.

- Resuspending cleaned templates in ~14-ul (i.e., 70% * 20 ul – i.e., remaining PCR volume) of buffer should allow you to aliquot 3 ul of cleaned templates for sequencing. (Note: consider running a new gel to verify concentrations.)

- Templates for DNA Sequencing:

- Clean entire PCR volumes, resuspend in TVLE (see “Choice of primer resuspension buffer?”), and aliquot templates into wells for individual sequencing reactions.

- PCRs with high primer input or “difficult-to-remove” primers:  If suspected, repeat cleaning & resuspension steps to ensure sequencing reactions will contain just 1 primer (i.e., no detectable levels of opposite PCR primer in sequencing reactions).  Before recleaning, vortex resuspended samples briefly, touch-spin samples, and  resuspend (>15 min).

- Miscellaneous notes:

- Better results are obtained with plates (or 8-strip tubes in 0.2-ml 96-place rack) and a plate centrifuge as described in the “Plates & Strip-tubes” version of the protocol on the Core website.

- These speeds & times were designed for a low-speed plate-centrifuge; you can reduce spin times with the higher speeds achievable in traditional bench-top models.

- Store EtOH solutions in a low-humidity environment (e.g., fridge or freezer) to prevent absorption of water from the air; your DNA won’t precipitate if the % of EtOH drops too low.

- Steps 6 & 9: It is important to fully remove the ethanol as any residual ethanol may contain high levels of the original PCR primers; if present in a sequencing reaction, the residual primers may dramatically reduce read quality.

1)	Remove tubes from thermocycler; spin tubes to consolidate all fluids at the bottom.  Transfer contents to tubes large enough to fit in your centrifuge rotor.

2)	For 10 ul samples, add 5 μl EDTA (70-150 mM, pH 8), ensuring droplet enters sample before Step 3.

– Alternative #1: For 20 μl samples, use 5 μl @ 150 mM EDTA & increase EtOH volumes, including wash, to 60 μl.

– Alternative #2: SpeedVac samples dry; add 15 EDTA (30-40 mM); incubate ~10 min; seal, vortex and touch-spin.

3)	Add 30-33 μl 100% EtOH to each tube; cap & shake tubes to shift EtOH top-to-bottom (repeat 4X).

4)	Incubate samples @ room temperature (>4 min – RT works better than 4oC).

5) 	Orient tubes identically in rotor so DNA will pellet in same location in each tube; spin tubes (minimum speed = 2500 RCF, 20oC, 15 min).  Proceed to Step 6 immediately; otherwise, first briefly re-spin tubes (2 min).

6)	Remove supernatants by either (A) aspirating with a pipette tip (sliding pipette tip down wall opposite pellet) or (B – for large volumes) using the ‘dump-blot-spin-aspirate’ process (i.e., dumping supernatant; blotting tubes on a clean Kimwipe; centrifuging tubes to collect residual EtOH; and, aspirating remaining EtOH).

7)	Add 30-33 μl 70% EtOH to each tube; cap tubes; shake tubes as in Step #3.

8)	Spin tubes as in Step 5, but centrifuge for only 5 min (to ensure pellet is stuck to tube).

9)	Remove supernatant by aspiration; consider doing a second 70% wash to remove more residual PCR primers.

10)	Dry tubes (caps open) to remove traces of EtOH (RT, >15 min; 70oC, >1 min; or SpeedVac); then, resuspend DNA in sufficient TVLE to achieve desired concentrations.  (e.g., For sequencing 500-750 bp fragments, plan to use 1-5 ul of template, giving a total of 2-6 ng template/reaction; see “How much DNA to use in a sequencing reaction?”).