ProNex Beads:  Dual Size-selection to target fragments of 100-700+ bp

(Use Low-Retention Filter tips for everything except Washes – 03 June 2021)

- Bring ProNex resin to room temperature for 30-60 min.

- Edit Excel Worksheet Table (below) for sample volume (for different size-selections, also edit ProNex:sample ratios).

- To aid in bead retention on the magnet, add 5 ul of dilute Tween-20 (0.___% in TVLE [10 mM Tris, 0.005 mM EDTA) to bring sample to ~0.005% Tween-20 in Step 8 (2nd Size-Selection).

- Transfer samples to 1.5-ml Lo-Bind Eppendorf tubes.

- Resuspend ProNex resin (vortex hard, 10 s).

- 1st Size-Selection:  Remove large fragments (>700-1000 bp) using a 0.9X Bead:Sample ratio.

  - For ___ ul sample (original volume + 5 ul dilute Tween-20), add ___ ul ProNex beads.

  - Mix by pipetting (10X); if needed, gently flick tubes to gather liquid at bottom of tube.

  - Incubate 10 min (RT).

  - Touch-spin to settle contents;  put on Magnetic stand (10 min – enhances bead removal vs. recommended 2 min).

- Transfer supernatants to new Lo-Bind tubes.

  - Optional:  Store beads (add 20 ul TVLE; touch-spin) in case they kept smaller fragments in error.

  - Supernatants should contain desired fragment population, plus fragments <100-bp.

- 2nd Size-Selection:  To retain >100-bp fragments on beads, add more ProNex to bring total ProNex:Sample ratio to ~3X.

  - To each sample (now ~___ ul), add ___ ul ProNex beads (2nd Size-Sel. Bead volume – Initial Bead volume).

  - Mix by pipetting (10X); if needed, gently flick tubes to gather liquid at bottom of tube.

  - incubate 10 min (RT).

  - Touch-spin to settle contents; place on Magnetic stand (10 min).

- Remove supernatant.

  - Optional:  In case desired fragments were not retained on beads, store supernatant in new Lo-Bind tubes.

  - Desired fragment population (~100-bp to ~1000-bp) should be on beads.

- Leaving sample on magnet,

  - Add 200 ul Wash Buffer (can use 1 tip for all tubes).

  - Incubate 1 min; then, slowly rotate tubes 2X to expose beads internal to ‘glob’.

  - Incubate an additional 2 min to fully pellet beads on magnet.

  - Remove and discard Wash Buffer.

- Repeat Step 9 (but, repeat Step 10 only if ‘small fragment’ population in sample is extremely high).

- Air dry sample (minimum, 5’; up to 60’ for maximum EtOH removal –  w/o losing HMW DNA).

- Remove samples from Magnet.

- Add 20 ul EB (10 mM Tris [pH 8.5], 0.005% Tween-20).

  - Resuspend resin by vortexing or pipetting.

  - Incubate samples (RT, 2 min)

  - Lightly vortex & touch-spin samples.

  - Incubate samples (RT, 3 min).

  - Put samples on Magnet (10 min;  Note –  If the beads do not separate well from the supernatant with 20 ul of EB, add 10 ul more of EB and repeat Steps 14a-e).

  - Transfer supernatants to new 1.5-ml Lo-Bind tubes.

- Put samples back on magnet.

  - After 15 min, examine samples for residual beads.

  - If needed, repeat the transfer of supernatants to new Lo-Bind tubes.

- Fragments should be roughly between 100 bp and 700+ bp in the final library.